Lab Session 03

OBJECT

Learning Arithmetic and logic instructions and flow control: 8086 Assembler

THEORY

Arithmetic and Logic Instructions

Most Arithmetic and Logic Instructions affect the processor status register (or Flags)



As you may see there are 16 bits in this register, each bit is called a flag and can take a value of 1 or 0.

- Carry Flag (CF) - this flag is set to 1 when there is an unsigned overflow. For example when you add bytes 255 + 1 (result is not in range 0...255). When there is no overflow this flag is set to 0.

- Zero Flag (ZF) - set to 1 when result is zero. For none zero result this flag is set to 0.

- Sign Flag (SF) - set to 1 when result is negative. When result is positive it is set to 0. Actually this flag take the value of the most significant bit.

- Overflow Flag (OF) - set to 1 when there is a signed overflow. For example, when you add bytes 100 + 50 (result is not in range -128...127).

- Parity Flag (PF) - this flag is set to 1 when there is even number of one bits in result, and to 0 when there is odd number of one bits. Even if result is a word only 8 low bits are analyzed!

- Auxiliary Flag (AF) - set to 1 when there is an unsigned overflow for low nibble (4 bits).

- Interrupt enable Flag (IF) - when this flag is set to 1 CPU reacts to interrupts from external devices.

- Direction Flag (DF) - this flag is used by some instructions to process data chains, when this flag is set to 0 - the processing is done forward, when this flag is set to 1 the processing is done backward.

There are 3 groups of instructions.

First group: ADD, SUB,CMP, AND, TEST, OR, XOR

These types of operands are supported:

REG, memory
memory, REG
REG, REG
memory, immediate
REG, immediate

REG: AX, BX, CX, DX, AH, AL, BL, BH, CH, CL, DH, DL, DI, SI, BP, SP.

memory: [BX], [BX+SI+7], variable, etc...

immediate: 5, -24, 3Fh, 10001101b, etc...

After operation between operands, result is always stored in first operand. CMP and TEST instructions affect flags only and do not store a result (these instruction are used to make decisions during program execution).

These instructions affect these flags only:
       CF, ZF, SF, OF, PF, AF.

- ADD - add second operand to first.

- SUB - Subtract second operand to first.

- CMP - Subtract second operand from first for flags only.

- AND - Logical AND between all bits of two operands. These rules apply:

1 AND 1 = 1
1 AND 0 = 0
0 AND 1 = 0
0 AND 0 = 0

As you see we get 1 only when both bits are 1.

- TEST - The same as AND but for flags only.

- OR - Logical OR between all bits of two operands. These rules apply:

1 OR 1 = 1
1 OR 0 = 1
0 OR 1 = 1
0 OR 0 = 0

As you see we get 1 every time when at least one of the bits is 1.

- XOR - Logical XOR (exclusive OR) between all bits of two operands. These rules apply:

1 XOR 1 = 0
1 XOR 0 = 1
0 XOR 1 = 1
0 XOR 0 = 0

As you see we get 1 every time when bits are different from each other.

Second group: MUL, IMUL, DIV, IDIV

These types of operands are supported:

REG
memory

REG: AX, BX, CX, DX, AH, AL, BL, BH, CH, CL, DH, DL, DI, SI, BP, SP.

memory: [BX], [BX+SI+7], variable, etc...

MUL and IMUL instructions affect these flags only:
       CF, OF
When result is over operand size these flags are set to 1, when result fits in operand size these flags are set to 0.

For DIV and IDIV flags are undefined.

- MUL - Unsigned multiply:

when operand is a byte:
AX = AL * operand.

when operand is a word:
(DX AX) = AX * operand.

- IMUL - Signed multiply:

when operand is a byte:
AX = AL * operand.

when operand is a word:
(DX AX) = AX * operand.

- DIV - Unsigned divide:

when operand is a byte:
AL = AX / operand
AH = remainder (modulus). .

when operand is a word:
AX = (DX AX) / operand
DX = remainder (modulus). .

- IDIV - Signed divide:

when operand is a byte:
AL = AX / operand
AH = remainder (modulus). .

when operand is a word:
AX = (DX AX) / operand
DX = remainder (modulus). .

Third group: INC, DEC, NOT, NEG

These types of operands are supported:

REG
memory

REG: AX, BX, CX, DX, AH, AL, BL, BH, CH, CL, DH, DL, DI, SI, BP, SP.

memory: [BX], [BX+SI+7], variable, etc...

INC, DEC instructions affect these flags only:
       ZF, SF, OF, PF, AF.

NOT instruction does not affect any flags!

NEG instruction affects these flags only:
       CF, ZF, SF, OF, PF, AF.

- NOT - Reverse each bit of operand.

- NEG - Make operand negative (two's complement). Actually it reverses each bit of operand and then adds 1 to it. For example 5 will become -5, and -2 will become 2.

Program Flow Control

Controlling the program flow is a very important thing, this is where your program can make decisions according to certain conditions.

- Unconditional Jumps

The basic instruction that transfers control to another point in the program is JMP.

The basic syntax of JMP instruction:

JMP label

To declare a label in your program, just type its name and add ":" to the end, label can be any character combination but it cannot start with a number, for example here are 3 legal label definitions:

label1:
label2:
a:

Label can be declared on a separate line or before any other instruction, for example:

x1:
MOV AX, 1

x2: MOV AX, 2

Here is an example of JMP instruction:

MOV AX, 5 ; set AX to 5. MOV BX, 2 ; set BX to 2. JMP calc ; go to 'calc'. back: JMP stop ; go to 'stop'. calc: ADD AX, BX ; add BX to AX. JMP back ; go 'back'. stop:

Of course, there is an easier way to calculate the sum of two numbers, but it's still a good example of JMP instruction.
As you can see from this example JMP is able to transfer control both forward and backward. It can jump anywhere in current code segment (65,535 bytes).

Tasks:

  - Create a program using the INC,DEC instructions.

  - Run the following program and explain its execution

MOV    AX, 5          ; set AX to 5.

MOV    BX, 2          ; set BX to 2.

JMP    calc           ; go to 'calc'.

back:  JMP stop       ; go to 'stop'.

calc:

ADD    AX, BX         ; add BX to AX.

JMP    back           ; go 'back'.

stop:

  - Write an 8086 Assembly program that calculates the following expression using only arithmetic instructions (ADD, SUB, MUL, DIV, INC, DEC):

((25+15)×4−20)÷5

Store the final quotient and remainder in the appropriate registers.

  - Write an 8086 Assembly program that intentionally generates each of the following flags at least once during execution:

- Carry Flag (CF)

- Zero Flag (ZF)

- Sign Flag (SF)

- Overflow Flag (OF)

- Parity Flag (PF)

- Auxiliary Flag (AF)